# Manufacturing Condition Ontology V1.0

Bryce Barrineau

Table of Contents

Purpose	4

1.	Product Defects	5

1.1	Lens Presentation Defects	5

Missing Lens	5

Multiple Lens	5

1.2	Foreign Matter & Debris Defects	5

Fiber	5

Extraneous Polymer	5

Foreign Matter	5

HEMA Fragment	5

2.	Process Conditions	6

2.1	Bubble Conditions	6

Bubble	6

Stretched Bubble	6

Irregular Bubble	6

Bubble Cluster	6

Bubble On Edge	6

Bubble Over 123 Mark	6

Bubble Scatter	6

2.2 Primary Package Conditions	6

Wet Package	6

3.	Product Presentation Conditions	7

Folded Lens	7

Lens Off Center	7

4.	Imaging & Inspection Conditions	8

4.1	Package Configuration	8

Missing Primary Package	8

Primary Package Misalignment	8

Cavity Off Center	8

4.2	Obstructions	8

Field Of View Obstructed	8

HEMA Obstruction	8

Low Dose Obstructing ROI	8

4.3	Equipment Conditions	8

Dirty Camera	8

Dirty Strobe	8

Hierarchical View	9

Annotation Specification	10

Image Level Labels (TAG)	10

Object-Level Labels (Bounding Box)	10

Object Sizes	11

# Purpose

This ontology defines all conditions that may be identified within FMD images. Conditions are organized by their semantic meaning, rather than annotation method or business impact.

The hierarchy supports:

- Dataset organization

- Annotation standards

- Model development

- Model validation

- Performance reporting

- Future ontology expansion

The hierarchy does not imply severity, criticality, or disposition.

# Product Defects

Conditions representing true product defects

## Lens Presentation Defects

### Missing Lens

This tag is to be used when there is no lens present in the image, but the primary package is visible, centered, and properly seated within its cavity.

### Multiple Lens

This tag is to be used when there is more than one lens in the image. This tag should be used even if one or both lenses are partial (i.e. cut in half or broken in half).

## Foreign Matter & Debris Defects

### Fiber

Fibers are short hair-like structures. They appear most often as curved or jagged structures that may curl back onto themselves.

### Extraneous Polymer

Extraneous polymer is excess lens material that typically appears stringy and is separate from the lens. Extraneous polymer may look thick or thin and can be mistaken for debris, exterior excess, or tears.

### Foreign Matter

The foreign matter (FM) category encompasses any type of debris that cannot be classified as HEMA, Extraneous Polymer, or Fiber. It can exist anywhere within the illuminated bowl and can range in size, shape, and color.  FM in this category are identifiable by their irregular shape and dark silhouettes.

### HEMA Fragment

HEMA is the excess monomer that is removed from the lens during the demold process. It can appear under the FMD camera as an entire ring or just a segment of the ring. It is commonly confused with extraneous polymer but can be distinguished by its ribbed pattern. This tag refers to cases where the ring has been broken into smaller chunks and presents as an object that can be bounded rather than obstructing a large swathe of the image.

# Process Conditions

Conditions associated with manufacturing processes that are not necessarily defects

## Bubble Conditions

### Bubble

Bubbles can appear in different forms, such as oblong or low-contrast shapes. Attributes label bubbles by size and lens location. Each attribute type is detailed in the following sections.

Bubbles are most often confused with debris. Unlike debris, bubbles have very round edges, even when they are oblong.

Bubbles less than 5x5 pixels should not be annotated with the bubble label. Bubbles smaller than 5x5 pixels can only be annotated as bubble clusters or bubble scatter when they appear as such.

### Stretched Bubble

Bubbles that have taken on elongated shape due to being stretched by forces acting on the lens-solution interface

### Irregular Bubble

Bubbles that have taken on dark, button-capped [BB5.1]shape due to surface tension effects on the lens-solution interface.

### Bubble Cluster

The cluster attribute is used when 2 or more bubbles are so close together that they appear to be touching. This is different than a bubble scatter in that the bubbles are more clumped together and a box can be drawn around the cluster separating it from other single bubbles or clusters.

### Bubble On Edge

This label is used when bubbles have contact with the edge or the edge-shadow.

### Bubble Over 123 Mark

This label is applicable when a bubble appears over the “123 mark.” Bubbles overlapping with the 123-mark often can become elongated or obscured but will always keep their rounded edges. Any bubble, regardless of its shape, size, or contrast level, that is situated near the 123-mark should be given this label.

### Bubble Scatter

Bubble Scatter presents as a field of many small bubbles beneath the lens surface, creating significant optical interference that impedes accurate inspection. This phenomenon differs from individual bubble obstructions by creating a diffuse pattern of interference across the field of view.

## 2.2 Primary Package Conditions

### Wet Package

This occurs when there is saline under or on the edge of the package. These sometimes present similarly to bubbles, but they are saline bubbles rather than air bubbles and therefore have more of a gradient across the surface, often with an irregular shape.

# Product Presentation Conditions

Conditions describing product positioning or presentation

### Folded Lens

This tag is to be used when a lens is folded over onto itself.

### Lens Off Center

This tag is to be used when the contact lens has shifted in solution, crossing over the solution package interface. This is commonly confused with Low Dose but is only applicable if the bowl is full of solution and the lens is still making contact with the edge of the solution (due to the lens not being centered).

# Imaging & Inspection Conditions

## Package Configuration

### Missing Primary Package

This tag is to be used when there is no primary package in-frame.

### Primary Package Misalignment

This tag is to be used when there is more than one primary package visible in-frame, or a package is not correctly seated in its cavity – preventing the FMD system from capturing the entire region of interest.

### Cavity Off Center

This occurs when the trigger and the pallet are out of sync, preventing the FMD system from capturing the entire region of interest. This is commonly confused with Package Misalignment but is only applicable if the package is clearly seated correctly.

## Obstructions

### Field Of View Obstructed

This tag is to be used When an opaque object blacks out a portion of the image

### HEMA Obstruction

HEMA is the excess monomer that is removed from the lens during the demold process. It can appear under the FMD camera as an entire ring or just a segment of the ring. It is commonly confused with extraneous polymer but can be distinguished by its ribbed pattern. This tag specifically refers to cases where the artifact is largely intact and obscures a large region of the image.

### Low Dose Obstructing ROI

This tag is to be used when the Lens-Solution Interface and the Solution-Package Interface overlap due to an inadequate saline dose.

## Equipment Conditions

### Dirty Camera

If the camera appears to be dirty, this tag should be applied. A dirty camera differs from a dirty strobe as the debris is more focused when compared with dirty strobe and is typically the result of dried saline water.

### Dirty Strobe

Dirty strobe occurs when saline or other substances build up on the cover between the strobe and the bottom of the package passing over it. This manifests in the FMD image as out-of-focus and dark marks that tend to “creep in” from the edges of the package.

# Hierarchical View

Manufacturing Image Condition Ontology

│

├── Product Defects

│ 	├── Lens Presentation Defects

│ 	│ 	├── Missing Lens

│ 	│ 	└── Multiple Lens

│ 	│

│ 	└── Foreign Matter Defects

│ 		├── Fiber

│ 		├── HEMA Fragment

│ 		├── Extraneous Polymer

│ 		└── Foreign Matter

│

├── Process Conditions

│ 	├── Bubble Conditions

│ 	│ 	├── Bubble

│ 	│ 	├── Stretched Bubble

│ 	│ 	├── Irregular Bubble

│ 	│ 	├── Bubble Cluster

│ 	│ 	├── Bubble On Edge

│ 	│ 	├── Bubble Scatter

│ 	│ 	└── Bubble Over 123 Mark

│ 	│

│ 	└── Package Conditions

│ 		└── Wet Package

│

├── Product Presentation Conditions

│ 	├── Folded Lens

│ 	└── Lens Off Center

│

└── Imaging & Inspection Conditions

│	├── Package Configuration

│ 	├── Missing Primary Package

│ 	├── Multiple Primary Package

│ 	├── Package Off Center

│ 	└── Cavity Off Center

│

├── Obstructions

│ 	├── Field Of View Obstructed

│ 	├── HEMA Obstruction

│ 	└── Low Dose Obstructing ROI

│

└── Equipment Conditions

├── Dirty Camera

└── Dirty Strobe

# Annotation Specification

## Image Level Labels (TAG)

- Missing Lens

- Multiple Lens

- Missing Primary Package

- Primary Package Misalignment

- Cavity Off Center

- Field Of View Obstructed

- HEMA Obstruction

- Low Dose Obstructing ROI

- Folded Lens

- Lens Off Center

- Dirty Camera

- Dirty Strobe

- Bubble Scatter

## Object-Level Labels (Bounding Box)

- Fiber

- Extraneous Polymer

- Foreign Matter

- HEMA Fragment

- Bubble

- Stretched Bubble

- Irregular Bubble

- Bubble Cluster

- Bubble On Edge

- Bubble Over 123 Mark

- Wet Package

# Object Sizes

Note: Statistics still being gathered for Stretched Bubble as of 2026-08-25

|  | Width |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Label | count | mean | std | min | 25% | 50% | 0.75 | max |
| Bubble | 1298 | 23 | 56.6 | 3.4 | 6.2 | 7.8 | 10.6 | 394.3 |
| Bubble Cluster | 154 | 38.7 | 80.8 | 5.1 | 11.5 | 17.7 | 32.6 | 720.1 |
| Bubble Irregular | 46 | 92.2 | 46.2 | 21.5 | 50.4 | 93.4 | 122 | 210.1 |
| Bubble On 123 Mark | 79 | 7.1 | 7.7 | 2.7 | 4.5 | 5.2 | 6.7 | 67.4 |
| Bubble On Edge | 90 | 13.5 | 26.6 | 4.4 | 6.3 | 8.6 | 12.8 | 256 |
| Extraneous Polymer | 17 | 199.5 | 226.4 | 30.4 | 61.4 | 120.8 | 198.7 | 926.6 |
| Fiber | 62 | 327.4 | 309.5 | 8.3 | 74.6 | 231 | 486.5 | 1228.8 |
| Foreign Matter | 77 | 36.9 | 28.7 | 6 | 19.9 | 29.9 | 45.9 | 202.9 |
| HEMA Fragment | 8 | 309.3 | 161.3 | 101.2 | 194.5 | 315.1 | 415.7 | 551.3 |
| Wet Package | 189 | 101.3 | 89.1 | 11.2 | 46.7 | 79.7 | 120.2 | 678.5 |

|  | Height |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Label | count | mean | std | min | 25% | 0.5 | 0.75 | max |
| Bubble | 1298 | 21.9 | 52.6 | 3.4 | 6 | 7.5 | 10.4 | 351.8 |
| Bubble Cluster | 154 | 33.6 | 74.7 | 4.5 | 11.7 | 18.3 | 29.4 | 683.6 |
| Bubble Irregular | 46 | 66 | 32.8 | 17.1 | 45.6 | 60.9 | 81.3 | 191.5 |
| Bubble On 123 Mark | 79 | 7 | 6 | 2.8 | 4.4 | 5.5 | 6.9 | 48.4 |
| Bubble On Edge | 90 | 13.8 | 20 | 3.7 | 5.9 | 10.4 | 14.7 | 187 |
| Extraneous Polymer | 17 | 133.2 | 84.1 | 18.8 | 65.1 | 131.1 | 206.5 | 260 |
| Fiber | 62 | 265.6 | 239.4 | 13.5 | 49.9 | 195.3 | 411.7 | 945.8 |
| Foreign Matter | 77 | 34 | 29.3 | 7.3 | 17.7 | 25.7 | 40.8 | 162.4 |
| HEMA Fragment | 8 | 327.2 | 186.2 | 112.9 | 162.4 | 321.9 | 433.9 | 644.3 |
| Wet Package | 189 | 68.2 | 59.6 | 6.5 | 24.6 | 44.5 | 100 | 332.1 |

| Label | Area |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble | count | mean | std | min | 0.25 | 0.5 | 0.75 | max |
| Bubble Cluster | 1298 | 3424 | 14855.7 | 11.6 | 37.2 | 57.9 | 110.7 | 120885.4 |
| Bubble Irregular | 154 | 7131.5 | 50535.4 | 28.2 | 137 | 339.3 | 1020.9 | 492275.5 |
| Bubble On 123 Mark | 46 | 7374 | 7317.1 | 398.7 | 2458.8 | 5638.2 | 9252.6 | 40231.7 |
| Bubble On Edge | 79 | 94.4 | 371.6 | 7.6 | 19.4 | 28.6 | 47.5 | 3261.9 |
| Extraneous Polymer | 90 | 701.1 | 5035 | 18.6 | 40.2 | 89.8 | 188.2 | 47876.2 |
| Fiber | 17 | 39076.9 | 61352.9 | 690.9 | 4489.8 | 11049.7 | 35894.4 | 240873.7 |
| Foreign Matter | 62 | 121536.5 | 149759.1 | 406.6 | 3181 | 49173.1 | 210953.2 | 617494.1 |
| HEMA Fragment | 77 | 1703.9 | 2996 | 57.9 | 437.1 | 647.2 | 1595.1 | 18392.8 |
| Wet Package | 8 | 82693.5 | 36912.9 | 29397.1 | 56805.3 | 88457.7 | 97729.2 | 145140 |
|  | 189 | 10596.6 | 20899.4 | 72.2 | 1494.1 | 2952 | 11531.4 | 149459.8 |